影像處理導論

宋其諭0510888

Project goal

Apply histogram-specification method to the image below, assume desired
histogram: nk=1248, 0≤k≤63 and 192≤k≤255 , nk=800, 64≤k≤191.
Determine the transformation function z = T(r) and the output image after
applying the transformation function to the image.

- Figures of the original and output histograms

在Histogram 的分佈中我們可以觀察到，如果 Histogram 分佈的峰值 靠近左側(靠近0 )，則代表這張影像偏暗；如果 Histogram 分佈的峰值 靠近右側(靠近255 )，則代表這張影像偏亮；如果 Histogram 分佈的峰值 非常集中在中間，則明暗的對比就不太明顯。，Histogram Equalization最理想分佈狀況就是平均分佈

- Figure of the output image after applying the histogram-specification scheme

從結果來看經過灰階均化後圖形效果較能看到細節的部分也較為清晰

- Table of transformation function to show the mapping from the input gray level r to the output gray level z

將做完轉換的灰階值存放在一個矩陣內

Output gray level:

0	0	0	0	0	0	0	0	0	0	0	0	1	1	1	2	2	3	4	6	8	10	13	16	20	24	28	31	34	37	40	43	46	49	52	55	58	61	63	67	71	75	79	82	86	90	93	97	101	104	107	111	114	117	120	124	127	130	133	136	139	141	144	146	148	151	153	155	157	159	161	163	165	166	168	170	171	172	174	175	177	178	179	180	181	183	184	185	186	187	188	189	191	192	192	193	194	194	195	195	196	197	197	198	199	199	200	201	201	202	203	203	204	204	205	206	206	207	207	208	208	209	209	210	210	211	211	212	212	213	213	214	214	215	215	215	216	216	217	217	217	218	218	218	219	219	219	220	220	220	221	221	221	222	222	222	222	223	223	223	224	224	224	225	225	225	225	226	226	226	227	227	227	228	228	228	228	229	229	229	229	230	230	230	230	231	231	231	232	232	232	232	233	233	233	233	234	234	234	234	234	235	235	235	235	235	235	235	236	236	236	236	236	236	236	236	237	237	237	237	237	237	237	237	237	237	237	238	238	238	238	238	238	238	238	238	238	238	238	239	239	239	239	239	239	239	239	239	239	239	240	240	240	240	243	255

- Source codes

本次實驗使用Matlab 軟體分析(含註解)

clc;

clear;

close;

a = imread('camellia (mono) 512x512.tif');                %Read the Image

b = size(a);

scale = 1;

shift = 0;

r = 0:255;                                          %Defining input pixels

d = round(scale*r)+ shift;                          %linear Transformation function

count = zeros(3,256);

spec = zeros(1,256);

ep = zeros(b);

transform = zeros(b);

%loop for spec gray level accumulation

for i= 0:255

if i>63 && i<192

spec(1,i+1)=spec(1,i)+800;

elseif i==0

spec(1,i+1)=1248;

else

spec(1,i+1)=spec(1,i)+1248;

end

end

%loop for count input image gray level

for i=1:b(1)                                        %loop tracing the rows of image

for j=1:b(2)                                    %loop tracing thes columns of image

t=(a(i,j)+1);                               %pixel values in image

ep(i,j)=d(t);                              %Making the ouput image using

count(1,ep(i,j))=count(1,ep(i,j))+1;     %counting

end

end

%loop for count input gray level accumulation

for i= 0:255

if i==0

count(2,i+1)=0;

else

count(2,i+1)=count(2,i)+count(1,i+1);

end

end

%loop for comparsion

for i=1:256                                        %loop tracing the  arrary count

for j=1:256                                   %loop tracing thes columns of image

if count (2,i)>spec(1,j)

count (3,i)=j;

end

end

end

%transform

for i=1:b(1)                                        %loop tracing the rows of image

for j=1:b(2)                                    %loop tracing thes columns of image

t=(a(i,j)+1);                               %pixel values in image

ep(i,j)=d(t);                              %Making the ouput image using

transform(i,j)=count(3,ep(i,j));     %counting

end

end

%Procedure for plotting histogram

hist1 = zeros(1,256);                               %prealocation space for input histogram

hist2 = zeros(1,256);                               %prealocation space for output histogram

for i1=1:b(1)                                       %loop tracing the rows of image

for j1=1:b(2)                                   %loop tracing the Columns of image

for k1=0:255                                %loop checking which graylevel

if a(i1,j1)==k1                         %match found at k1

hist1(k1+1)=hist1(k1+1)+1;          %increase the value at k1

end

if transform(i1,j1)==k1                        %for output image

hist2(k1+1)=hist2(k1+1)+1;

end

end

end

end

%Plotting input image output image and their respective histograms

subplot(2,2,1);

imshow(a);

title('original image');

subplot(2,2,3);

imshow(uint8(transform));

title('output image');

subplot(2,2,2);

plot(hist1);

subplot(2,2,4);

plot(hist2);

%Plotting input image output image

figure;

subplot(1,2,1);

imshow(a);

title('original image');

subplot(1,2,2);

imshow(uint8(transform));

title('output image');

%%%

figure;

plot(count(3,:));

xlabel('input gray level');

ylabel('output gray level');